1st Absorber in LTF

D. Errede, M. Haney

V = 9 liters

R = 15 cm

L = …   (Not an SFOFO design)

2nd Absorber

R = 11cm

L = 21cm

and will be in beam w/B-field

Temperature liquid H2  13.81 – 20.28K

Our operating range  17-20K

inlet    outlet

Pressure – 1.2 atm or 18psig

----------

Energy/muon pulse:

dE/dx _Al = 4.8247 MeV/cm

dE/dx_lH2 = 0.33059 MeV/cm

for L = 21 cm

Energy/muon = 6.9 MeV + 0.3 MeV  = 7.2 MeV

Energy/muon pulse = 7.2  10**12 MeV   assuming 10**12 muons/pulse

E = 1.15 Joules / pulse

P_instant = E/pulse/(t~10**-8)    = 1.15  10**8 Watts

where t = L_cell/0.9c  = 2.75m/0.9c

P_integrated = 1.152 Joules  15Hz     = 17 Watts

----rep rate

will complete this estimate later.

dQ/dT(lH2) = in Joules/mol-K (roughly)

Ed Black : flow rate estimate

1 volume changes per second is moving  10 liters   30cm/sec

30 cm diameter of second absorber.

1 Absorber

Number of Channels to Read Out

12          temp gauges/absorber including one near inlet and outlet flow positions

resolution to 1/10 degree K.

2       pressure transducers/absorber (cryogenic) 100 psi  w/lower bound of ¼ psi.

resolution dictated by backgrd noise.

2            flow neters

2             channels for photogrammetry

4          channels for laser (occlusion sensing)  (occlusion sensing)

2           piezo-electric transducers  (high frequency vibrations)  (Oxford)

2           strain measurements  (FISO not yet tried in cryo environment)

X       bolometry: unknown at this point.

Channels   and Cost of Sensors

12          temp gauges                      <= $100 each

2       pressure trans                          $1600 each

2            flow meters                         $1500 each (guess)

2       channels for photogrammetry         X

4          channels for laser                         X

2           piezo-electric transducers          $100 each

2           strain measurements                      $5k/each  for signal conditioning

X       bolometry: unknown at this point.

Total channels = 26

generic cost/channel is    $100-$1000/channel

will use $500/channel

26 x $500  = $13,000

Cost for sensors   ~ $18,000

Total     31k$